# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Waste removal truck	p 1 /…

Publication reference: HR-BA-ME00365 – RECIRCLE – PP5 - T1

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | • Chassis new and unused, chassis production year min. in 2024 • Cabin short – dailycab • 4 x 2 drive configuration • Total load capacity of the chassis min. 18 t • Steering shaft with mechanical support • Air-suspended drive axle, parabolic spring steering axle • Drive shaft differential lock • Load capacity of the driving axle min. 12 t • Stabilizers and shock absorbers on the steering and drive shafts • Engine, diesel, 6 cylinders in line volume min. 6.8 liters max. 7.5 liters • Maximum engine torque min. 1,050 Nm • Engine power min. 250 hp • Engine emission class min. EURO 6 • Automatic gearbox • Power take-off from the gearbox aligned with the needs of the superstructure • Motor brake with a power of at least 170 kW with the possibility of automatic activation via the brake pedal • Fuel filter with heater • Fuel pre-filter with water sensor • Air dryer with heating • Tires (6 + 1) front and rear dimensions min. 295/80R22.5 • Cabin color white • Passenger seats 1 + 2 • Audible signal for reversing • Air conditioner driven by a compressor from the engine • Central locking of the cabin • Main and wide-angle exterior mirrors with heating and electric adjustment • Factory tools, first aid, triangle, hydraulic jack • Tire filling hose min. 20 m long with pressure gauge • Disc brakes on all axles • ABS, ASR and ESP • Parking brake with locking of all wheels • Fuel tank min. 300 liters, with lid locking with the same key as the cabin and a strainer in the spout • Tank Ad Blue min. 35 liters with lock • Electric windows on the driver&#39;s and front passenger&#39;s doors • Tank cover with chain • Curbside mirror, right electrically adjustable with heating • EU front mirror • Air-supported driver&#39;s seat • All seats with seat belts • Digital tachograph •Travel computer • Front underride protection • Lights in the door to illuminate the surroundings for easy entry and exit • Front and rear fog lights • Mechanical battery disconnect switch • Factory radio with speakers • Maintenance-free batteries 12V min. 175 Ah • Vehicle and driving information on the display • Brake pad thickness display, with brake pad wear warning • Regulation of the length of the light beam • Double halogen headlights • Daytime running lights LED • Digital instrument panel dimensions min. 12.0 inches TECHNICAL SPECIFICATION OF REQUESTED SUPERSTRUCTURE  Body upfit consists of a body and its subframe mounted on a truck chassis  Tailgate with compaction mechanism , discharging/ ejection plate inside the body, bin lifter device, hydraulic and electrical components and their installation on garbage compactor  Capacity (m3): min 15m3 excluding tailgate ( rear hopper)  Body Floor: St-52 / min 4mm  Body Sides: St-52 / min 4mm  Body side walls are smooth type and convex shaped without any vertical profile/ribs and traverces on side walls  Body Roof: St-52 (S355J2) / min 4mm Ejection/Discharging Plate:  Discharging plate moves on the rail welded above from the floor to the inner sides of the body  Discharging plate moves via double acting telescopic cylinder  It is kept at the rear of the body when it is empty and moves automatically to the front of body by pressure valve as the garbage is started to packed and pressed inside body  Dirty Water Tank: Dirty water tank in front bottom of the body with min capacity 70 liters  Tailgate is connected to the body with top hinges and opened upwards by means of two units double acting hydraulic cylinders  Compaction Ratio: The compaction ratio (volumetric) varies between min 3:1 to 6:1 according to the characteristic and due to this density of the garbage used in the area and garbage compactor operating pressure  Rear Hopper Capacity (m3): min 5m3 (cbm)  Sealing: With special design leak-proof rubber seal between tailgate and the body  Locking: Hydraulic locking by means of reverse action of two hydraulic tailgate-lifting cylinders  Safety Props: tailgate is provided with safety props (manual) between the body and tailgate for safety purposes during maintenance  Safety Valve: Equipped with antidropping safety valve on tailgate hydraulic lines to prevent sudden falls Tailgate (Hopper) Steel thickness (mm):  Tailgate (rear hopper) floor: St52 / min 6 mm  Tailgate floor extension (opposite wall) inside the body: St52 / min. 8 mm  Tailgate (rear hopper) side walls (under compaction plate): St52 / min. 6 mm  Suitable to 1100-liter metal and plastic bins by steel arms + 120-240-360-770 (800) liter plastic container lifting mechanism (from one side control)  Container lifting device is activated/controlled by manually via hand levers  Tailgate control (opening and closing) and ejection plate control (forward and backward movement) with two button control at same time for closing tailgate  Compaction mechanism - System can be operated by electrically (automatically) via push buttons + Mechanical (manual) via hand lever(s)  Container lifting device is activated/controlled by manually via hand levers  Safety devices: Equipped with safety devices on tailgate hydraulic lines to prevent sudden falls  Emergency stop valve at both sides, and a rescue system in one side  Oil tank capacity: min 115 liter capacity  Equipped with return filter,oil indicator and tank lid  Working pressure (bar): min 180 ± 10 bar  Hydraulic cylinder: min 10 units hydraulic cylinder  Telescopic cylinder with min 4 extensions  All the hydraulic cylinders are double-acting cylinder  The dimensions of cylinders are designed to accomplish the stipulated cycle times and compaction ratio  Hydraulic hoses: Comply to SAE100 – R1,R2 or R3 ,R4 or R9 depending on location ,purpose and dimension of hoses.  Hydraulic pump: P.T.O driven via gearbox  Hydraulic tandem pump included  Min 2 units rotating lights included: 1 is in front of the body and other is on tailgate 1 unit working light mounted on tailgate for lightning working area  Two units foldable, anti-sliding and Height adjustable type rear foot boards for crew to stand on  Side bicycle protection bar/barrier included  One elliptical handle at each side  ¾ &quot; pipe handle for the crews to grasp  Rear camera and monitor included (appr. size 5-6’’)  Bin upper lid opener bar (catcher bar) activated by 1 x pneumatic cylinder  Access door in front of the body (the one that prevents access inside the body behind the ejection plate)  Plastic mudguards  Electrical system is separated from the circuit of the truck Washing (disinfection system) for rear door and containers with a spray gun :  The rear door and containers washing and disinfection system is located near the rear door and is an integral part of the superstructure, it allows the operator to immediately wash and/or disinfect the rear door and/or containers with a spray gun after work  Vessel for liquid used for washing and disinfection resistant to agents to be used for disinfection of rear doors and containers  Painting - All steel parts surfaces free of rust and oil residue by sand blasting , then will be applied with two primers and completely final painted in white (same color as a cabin of vehicle) Electronic control unit ( ECU ) – CAN BUS system - Must contains the following features:  Switching on/off beacon and work lamps, tracking status of the lamps by control panel.  Tracking status of PTO on the screen and saving total working time of the PTO  Slide and bucket can be operated via the control panel, the number of compressions is saved, and the warning about lubrication can be displayed when the set number of compressions is reached  The sliding curtain can be controlled via the control panel  The number of garbage unloading is saved, and the desired warning can be displayed when the set number of unloading is reached  Information that the scale is leading can be displayed on the screen  Container lift control can be controlled via the control panel, the number of operations is saved, the desired warning can be displayed when the set number of lifting is reached  The rear cover can be opened via the control panel, it can be displayed on the screen, and warning and / or blocking can be done if the driver wants to drive  The compression system can be selected via the control panel as continuous mode, single mode, and the selection can be displayed on the screen  When a fault occurs in the system, it can be seen in which region the component is located  Alarms from the system are saved  It can be protected by the menus password reached via the monitor  The return filter pollution warning can be displayed and, if desired, operation can be stopped after a certain period of time  The system pressure can be continuously displayed, warning when it reaches a certain value, system can stop after certain value  The oil temperature can be continuously displayed, warning when reaching a certain value, the system may stop after a certain value  The warning and / or system may stop when the oil level falls below the specified value  When the maintenance flaps are opened, the system can be stopped and the cause can be displayed on the screen  The warning given by the overload sensor to be placed on the axle can be displayed on the screen and the system can be stopped if requested  Information can be displayed on the screen to indicate that the person is human on the stage, a signal can be sent to stop the vehicle, and a signal to reverse the maneuver can be sent  If the system is canceled while there is a man on the step, the system will restart with the penalty delay until the time when the PTO will be put into service again  The rear view camera image can be viewed separately or on the same monitor as the system  If there is an additional cover allowing automatic operation and if it is on, the system can be operated manually, these can be displayed on the screen  All cables assembled on a central unit on the back cover will be transmitted to the body via a single line (CAN-Bus)  The body will also be transmitted via a single line to the control panel in the truck cabin in the second unit (CAN Bus)  There will be emergency stop buttons inside the cabin, front and rear, which can be displayed on the screen if it is pressed Technical details of top crane for emptying of underground and semi underground containers  Model: ……………………….  Lifting Capacity: min 9.5 (ton.meter)  Standard Reach: min 7.5 (m)  Hydraulic Extension: min 3.5 (m)  Rotation: min 330° / degree  Rotation Torque: min 42 (Kn.m)  Working Pressure: min 175 (bar)  Pump Capacity: min 35 (lt.dk)  Approximate Weight: max 1500 (kg)  Length: min 4400 (mm)  Width: max 950 (mm)  Height: must be 650 - 700(mm)  Color: RAL 3020  Lifting moment: min 95 kNm (9.5 TON/M)  Manufactured according to EN12999: HC1 HD4/S2  Overload protection and emergency cut-off: Including Turntable with 2 slewing motors, with rotary distributor and slip ring body: 1 slewing motors with slew drive, (without rotary distributor and slip ring)  1 work light - led type mounted on outer boom  Load holding valves mounted on slewing, main, outer boom and telescopic cylinder  Radio remote control: with min 6 function  Control valve mounted on crane columned  Hose package: Control valve mounted in front and top of the garbage compactor  Control valve fully proportional  High pressure filter: Including Hose equipment for two additional hydraulic functions: Instead of 2 function, there is 1 additional hydraulic functions  Oil tank: min 70 liters  Single hook system Other requirements: - General warranty period for the complete vehicle - 24 months from the day of vehicle collection and signing of the handover report - Authorization or confirmation for the chassis and superstructure from his authorized representative/distributor for the territory of Montenegro issued to the bidder that, in the procedure of this public procurement, he can offer their chassis or superstructure, serviced within the warranty and non-warranty period. - Confirmation/catalog or excerpt of catalog of the required technical characteristics of the chassis from the manufacturer of the chassis (or its authorized representative/distributor, for the territory of Montenegro) and superstructure certified and signed by the manufacturer of the superstructure with the mandatory indication of the model of the superstructure offered. - Proof for certified repairers - The bidder is obliged to provide for at least one person engaged in the business of providing service services for the type of vehicle he offers with proof of the training of the serviceman issued by the manufacturer of the goods; - It is obligatory to attach a catalog/excerpt from the catalog or a technical sheet of the CAN BUS system as proof that it meets all the characteristics required in the technical specification - It is obligatory to attach a signed technical specification/technical sheet for the top crane by the manufacturer as well as proof (manufacturers authorization) that the bidder can offer, service within the warranty and non-warranty period. |  |  |  |